ios Printer SDK API Reference Guide

V1.0.0

Contents

1. About this guide	4

1-1 Supported Platforms & Development Environment	4

1-2Models / interface list	4

1-3 Xcode static library use	4

2. Contant	7

2-1 Contant definition	7

2-1-1 character set	7

2-1-2International character set	8

2-1-3 Chinese encoding format	9

2-1-4 Barcode / image / text alignment	9

2-1-5 Select the print mode	9

2-1-6	Select HRI character printing position	9

2-1-7 Select and print the bar code system	10

2-1-8 Set character size	10

2-1-9 Real-time transmission printer ID	11

2-1-10 Real-time status transmit	11

2-1-11 Real-time printer request	11

2-1-12 Real-time pulse generator	11

2-1-13 Select the bitmap mode	12

2-1-14 Set / release underline mode	12

2-1-15 Select the font	12

2-1-16 Set / release 90 ° clockwise rotation	12

2-1-17 Set bitmap mode	12

2-1-18 Select HRI character font	13

2-1-19 Set / release inverse print mode	13

2-1-20 Barcode width	13

2-1-21 Select print direction in page mode	13

2-1-22 Set / release Bold	14

2-1-23 Select paper cutting mode	14

2-1-24 Set/Release invert printing mode	14

2-1-25 Port selection	14

3.RTPrinter command control class definition (reference)	15

3-1 Overview	15

3-2 Porperties	15

3-2-1 delegate	15

3-2-2 - (BOOL) InitPrinter	16

3-2-3 - (void) HT	16

3-2-4 - (void) LF	17

3-2-5 - (void) CR	17

3-2-6 - (void) Selftest Print	18

3-2-7 - (void) SetRightSpace:(Byte)Distance	18

3-2-8  - (void) SetAbsoluteprintPosition:(Byte)nL highByte:(Byte)nH	18

3-2-9  - (void) SetRelativeprintPosition:(Byte)nL highByte:(Byte)nH	19

3-2-10  - (void) Set Default Line Spacing	20

3-2-11 - (void) Set Line Spacing:(Byte)Line Spacing	20

3-2-12  - (void) Set Left Start Spacing:(Byte)nL highByte:(Byte)nH	20

3-2-13  - (void) Set Area Width:(Byte)nL highByte:(Byte)nH	21

3-2-14  - (void) Set Character Printmode:(Byte)Character Print mode	21

3-2-15  - (void) Set Underline:(Byte)UnderLine En	22

3-2-16  - (void) Add Bold:(Byte)BoldEn	22

3-2-17  - (void) SetCharacterFont:(Byte)Font	23

3-2-18  - (void) SetRotate:(Byte)RotateEN	23

3-2-19  SetAlignMode	24

3-2-20  - (void) SetInvert Print:(Byte)Invert ModeEn	24

3-2-21  - (void) Set Zoom:(Byte)Font Enlarge	25

3-2-22  - (void) Set BlackReverse Print:(Byte)BlackReverse En	26

3-2-23  - (void) OpenDrawer:(Byte)DrawerNumber startTime:(Byte)pulseStartTime endTime:(Byte)PulseEndTime	26

3-2-24  - (void) Cut Paper	27

3-2-25  - (void) partial Cut Paper	27

3-2-26     ① - (void) Feed And Cut Paper:(Byte)Cut Mode	28

②- (void) Feed And Cut Paper:(Byte)Cut Mode feed:(Byte)Feed Distance	28

3-2-27  Add Bar Code Print	28

3-2-28 Begin Print Image	30

3-3  Instance Methods	31

3-3-1  Begin	31

3-3-2 Excute	32

3-3-3 Import Data	32

3-3-4  Clear Data	32

3-3-5  btn Connect	33

3-3-6  btnPrint	33

3-3-7  btn Preferences Click	34

4.Agreement Involved	36

4-1 Outline	36

4-2 Instance Methods	36

4-2-1 connect Host	36

4-2-2  disconnect	37

5.Appendix	37

# 1. About this guide

This guide describes the SDK library functions for IOS application development.

## 1-1 Supported Platforms & Development Environment

- Platform

iOS 4.3 or higher

- Development environment

XCode 4.2.6 or higher

## 1-2Models / interface list

| Model | Ethernet | Wifi | Bluetooth |
| --- | --- | --- | --- |
| RP80 | ○ | × | × |
| RP80W | × | ○ | × |
| RPP-02 | × | × | ○ |

## 1-3 Xcode static library use

1-3-1 Add CFNetwork.framework and static library files libWifiPrinter_SDK.a

- Select "project file and target" option

- Select "Build Phasaes" label

- Select "Link Binary With Libaries" project, click on the "+" pop up boxes as follow, select CFNetwork.framework and static library files libWifiPrinter_SDK.a added to the project.

1-3-2 Add a static library appropriate header files

Add three static library header files to your project.:WifiPrinter.h, WifiPrinterDelegate.h, AsynSocket.h

# 2. Contant

SDK provides all defined constants used in WifiPrinter.h.

## 2-1 Contant definition

### 2-1-1 character set

The following is a list of character set supported , the default character set is RT_CS_CP437.

| Code | Value | Description |
| --- | --- | --- |
| RT_CS_CP437 | 0 | CP437【US，European standard】 |
| RT_CS_KATAKANA | 1 | KataKana【KataKana】 |
| RT_CS_CP850 | 2 | CP850【multilingual】 |
| RT_CS_CP860 | 3 | CP860【Portugal】 |
| RT_CS_CP863 | 4 | CP863【Canada - French】 |
| RT_CS_CP865 | 5 | CP865【Nordic】 |
| RT_CS_WP1251 | 6 | WCP1251【Cyrillic】 |
| RT_CS_CP866 | 7 | CP866【Slavic2】 |
| RT_CS_MIK | 8 | MIK【Slavic / Bulgarian】 |
| RT_CS_CP755 | 9 | CP755【Eastern Europe, Latvia2】 |
| RT_CS_CP862 | 15 | CP862【Hebrew】 |
| RT_CS_WCP1252 | 16 | WCP1252【 Latin1】 |
| RT_CS_WCP1253 | 17 | WCP1253【Greece】 |
| RT_CS_CP852 | 18 | CP852【Latin2】 |
| RT_CS_CP858 | 19 | CP858【Latin characters1+European languages】 |
| RT_CS_IRANII | 20 | Iran II【Persian】 |
| RT_CS_LATVIA | 21 | Latvia |
| RT_CS_CP864 | 22 | CP864【Arabic】 |
| RT_CS_ISO88591 | 23 | ISO-8859-1【Western Europe】 |
| RT_CS_CP737 | 24 | CP737【Greece】 |
| RT_CS_WCP1257 | 25 | WCP1257【Baltic】 |
| RT_CS_THAI | 26 | Thai |
| RT_CS_CP720 | 27 | CP720【Arabic】 |
| RT_CS_CP855 | 28 | CP855 |
| RT_CS_857 | 29 | CP857【Turkish】 |
| RT_CS_WCP1250 | 30 | WCP1250【Central Europe】 |
| RT_CS_CP775 | 31 | CP775 |
| RT_CS_WCP1254 | 32 | WCP1254【Turkish】 |
| RT_CS_WCP1255 | 33 | WCP1255【Hebrew】 |
| RT_CS_WCP1256 | 34 | WCP1256【Arabic】 |
| RT_CS_WCP1258 | 35 | WCP1258【Vietnamese】 |
| RT_CS_ISO88592 | 36 | ISO-8859-2【Latin2】 |
| RT_CS_ISO88593 | 37 | ISO-8859-3【Latin3】 |
| RT_CS_ISO88594 | 38 | ISO-8859-4【Baltic】 |
| RT_CS_ISO88595 | 39 | ISO-8859-5【Cyrillic】 |
| RT_CS_ISO88596 | 40 | ISO-8859-6【Arabic】 |
| RT_CS_ISO88597 | 41 | ISO-8859-7【Greek】 |
| RT_CS_ISO88598 | 42 | ISO-8859-8【Hebrew】 |
| RT_CS_ISO88599 | 43 | ISO-8859-9【Turkish】 |
| RT_CS_ISO885915 | 44 | ISO-8859-15【9Latin 9】 |
| RT_CS_THAI2 | 45 | 【Thai 2】 |
| RT_CS_CP856 | 46 | CP856 |
| RT_CS_CP874 | 47 | CP874 |

### 2-1-2International character set

The following is a list of supported international character set, the default character set is RT_ICS_CHINESE

| Code | Value | Description |
| --- | --- | --- |
| RT_ICS_USA | 0 | United States |
| RT_ICS_FRANCE | 1 | France |
| RT_ICS_GERMANY | 2 | Germany |
| RT_ICS_ENGLAND | 3 | British |
| RT_ICS_DENMARKI | 4 | Denmark I |
| RT_ICS_SWEDEN | 5 | Sweden |
| RT_ICS_ITALY | 6 | Italy |
| RT_ICS_SPAINI | 7 | Spanish I |
| RT_ICS_JAPAN | 8 | Japan |
| RT_ICS_NORWAY | 9 | Norway |
| RT_ICS_DENMARKII | 10 | Denmark II |
| RT_ICS_SPAINII | 11 | Spain II |
| RT_ICS_LATIN | 12 | Latin America |
| RT_ICS_KOREAN | 13 | Korea |
| RT_ICS_SLOVENIA | 14 | Slovenia |
| RT_ICS_CHINA | 15 | China |

### 2-1-3 Chinese encoding format

| Code | Value | Description |
| --- | --- | --- |
| RT_ENCODING_GBK | 0 | GBK encoding |
| RT_ENCODING_UTF_8 | 1 | UTF-8 encoding |
| RT_ENCODING_BIG5 | 3 | BIG5 traditional Chinese encoding |

### 2-1-4 Barcode / image / text alignment

| Code | Value | Description |
| --- | --- | --- |
| RT_ALIGNMENT_LEFT | 0 | Align Left |
| RT_ALIGNMENT_CENTER | 1 | Align Center |
| RT_ALIGNMENT_RIGHT | 2 | Align Right |

### 2-1-5 Select the print mode

| Code | Value | Description |
| --- | --- | --- |
| RT_FT_DEFAULT | 0x00 | default value |
| RT_FT_FONTA | 0x00 | Character font A（12*24） |
| RT_FT_FONTB | 0x01 | Character font B（9*17） |
| RT_FT_BOLD | 0x08 | Set bold mode |
| RT_FT_DHEIGHT | 0x10 | Set times the height mode |
| RT_FT_DWIDTH | 0x20 | Set times the height width mode |
| RT_FT_UNDERLINE | 0x80 | Set underline mode |

### 2-1-6	Select HRI character printing position

| Code | Value | Description |
| --- | --- | --- |
| RT_HRI_NONE | 0 | Do not print HRI character |
| RT_HRI_ABOVE | 1 | Above the barcode |
| RT_HRI_BELOW | 2 | below the barcode |
| RT_HRI_BOTH | 3 | Above and below the barcode |

### 2-1-7 Select and print the bar code system

| Code | Value | Number of Data | Range of Data |  |
| --- | --- | --- | --- | --- |
| RT_BCS_UPCA | 0x65 | 11<=n <=12 | 48<=d<=57 |  |
| RT_BCS_UPCE | 0x66 | 11<=n<=12 | 48<=d<=57 |  |
| RT_BCS_EAN13 | 0x67 | 12<=n<=13 |  | 48<=d<=57 |
| RT_BCS_EAN8 | 0x68 | 7<=n<=8 |  | 48<=d<=57 |
| RT_BCS_CODE39 | 0x69 | 1<=n<=255 |  | 48<=d<=57,65<=d<=90,32,36,37,43,45,46,47 |
| RT_BCS_ITF | 0x70 | 1<=n<=255(even number) |  | 48<=d<=57 |
| RT_BCS_CODABAR | 0x71 | 1<=n<=255 |  | 48<=d<=57,65<=d<=68,36,43,45,46,47,58 |
| RT_BCS_CODE93 | 0x72 | 1<=n<=255 |  | 0<=d<=127 |
| RT_BCS_CODE128 | 0x73 | 2<=n<=255 |  | 0<=d<=127 |

### 2-1-8 Set character size

- Character width setting

| Code | Value | Description |
| --- | --- | --- |
| RT_TS_1WIDTH | 0x00 | 1(general) |
| RT_TS_2WIDTH | 0x10 | 2 times the width |
| RT_TS_3WIDTH | 0x20 | 3 times the width |
| RT_TS_4WIDTH | 0x30 | 4 times the width |
| RT_TS_5WIDTH | 0x40 | 5 times the width |
| RT_TS_6WIDTH | 0x50 | 6 times the width |
| RT_TS_7WIDTH | 0x60 | 7 times the width |
| RT_TS_8WIDTH | 0x70 | 8 times the width |

- Character height settings

| Code | Value | Description |
| --- | --- | --- |
| RT_TS_1HEIGHT | 0x00 | 1（general） |
| RT_TS_2HEIGHT | 0x01 | 2 times the height |
| RT_TS_3HEIGHT | 0x02 | 3times the height |
| RT_TS_4HEIGHT | 0x03 | 4times the height |
| RT_TS_5HEIGHT | 0x04 | 5times the height |
| RT_TS_6HEIGHT | 0x05 | 6times the height |
| RT_TS_7HEIGHT | 0x06 | 7times the height |
| RT_TS_8HEIGHT | 0x07 | 8times the height |

### 2-1-9 Real-time transmission printer ID

| Code | Value | Description |
| --- | --- | --- |
| RT_ID_MODLE | 49 | Printer ID Model |
| RT_ID_NUMBER | 50 | ID Model |
| RT_ID_VERSION | 65 | Hardware version |
| RT_ID_FIRM | 66 | Company |
| RT_ID_PRINTERNAME | 67 | Printer Name |
| RT_ID_UARTNAME | 68 | Serial name |
| RT_ID_ENCODINGTYPE | 69 | Multi-byte character |

### 2-1-10 Real-time status transmit

| Code | Value | Description |
| --- | --- | --- |
| RT_STATE_PRINTER | 1 | Printer Status |
| RT_STATE_OFFLINE | 2 | Offline Status |
| RT_STATE_ERR | 3 | Error Status |
| RT_STATE_SENSOR | 4 | Continuous paper sensor status |

### 2-1-11 Real-time printer request

| Code | Value | Description |
| --- | --- | --- |
| RT_REQ_LINE | 1 | Recover from the error and resume printing from the error line |
| RT_REQ_CLEAR | 2 | Recover from the error after clearing the receive and print buffers |

### 2-1-12 Real-time pulse generator

| Code | Value | Description |
| --- | --- | --- |
| RT_CASHDRAWER_PIN_2 | 0 | Cash drawer connect pin 2 |
| RT_CASHDRAWER_PIN_5 | 1 | Cash drawer connect pin 5 |

### 2-1-13 Select the bitmap mode

| Code | Value | Description |
| --- | --- | --- |
| RT_DENSITY_SINGLE_8 | 0 | 8 - point single density |
| RT_DENSITY_DOUBLE_8 | 1 | 8 - point double density |
| RT_DENSITY_SINGLE_24 | 32 | 24 - Point single density |
| RT_DENSITY_DOUBLE_24 | 33 | 24 - Point Dual Density |

### 2-1-14 Set / release underline mode

| Code | Value | Description |
| --- | --- | --- |
| RT_UNDERLINE_DIS | 0 | Release underline mode |
| RT_UNDERLINE_1 | 1 | Set underline mode (1 point coarse) |
| RT_UNDERLINE_2 | 2 | Set underline mode (2 point coarse) |

### 2-1-15 Select the font

| Code | Value | Descr |
| --- | --- | --- |
| RT_FONT_A | 0 | Select font A（12*24） |
| RT_FONT_B | 1 | Select font B（9*17） |

### 2-1-16 Set / release 90 ° clockwise rotation

| Code | Value | Description |
| --- | --- | --- |
| RT_CLOCKWISE_DIS | 0 | Release 90 ° clockwise rotation mode |
| RT_CLOCKWISE_EN | 1 | Set 90 ° clockwise rotation mode |

### 2-1-17 Set bitmap mode

| Code | Value | Description |
| --- | --- | --- |
| RT_BMP_NORMAL | 0 | Normal (vertical 203.2dpi, level 203.2dpi) |
| RT_BMP_DOUBLEWIDTH | 1 | Times the width(vertical 203.2dpi, level 101.6dpi)） |
| RT_BMP_DOUBLEHEIGHT | 2 | Times the height(vertical 101.6dpi, level 203.2dpi) |
| RT_BMP_FOURTHTIMES | 3 | 4 times the size(vertical 101.6dpi, level 101.6dpi) |

### 2-1-18 Select HRI character font

| Code | Value | Description |
| --- | --- | --- |
| RT_HRI_FONTA | 0 | Font A（12*24） |
| RT_HRI_FONTB | 1 | Font B（9*17） |

### 2-1-19 Set / release inverse print mode

| Code | Value | Description |
| --- | --- | --- |
| RT_INVERSE_DIS | 0x00 | Inverse mode off |
| RT_INVERSE_EN | 0x01 | Inverse mode on |

### 2-1-20 Barcode width

| Code | Value | Description |  |  |
| --- | --- | --- | --- | --- |
|  |  | Multi-stage bar code（mm） | Binary barcode（mm） |  |
|  |  |  | Strips | Broad |
| RT_BARCODE_1 | 2 | 0.250 | 0.250 | 0.625 |
| RT_BARCODE_2 | 3 | 0.375 | 0.375 | 1.000 |
| RT_BARCODE_3 | 4 | 0.560 | 0.500 | 1.250 |
| RT_BARCODE_4 | 5 | 0.625 | 0.625 | 1.625 |
| RT_BARCODE_5 | 6 | 0.750 | 0.750 | 2.000 |

### 2-1-21 Select print direction in page mode

| Code | Value | Description |
| --- | --- | --- |
| RT_DIRECTION_LEFT_RIGHT | 0 | Print from left to right |
| RT_DIRECTION_BOTTOM_TOP | 1 | Print from the bottom to top |
| RT_DIRECTION_RIGHT_LEFT | 2 | Print from right to left |
| RT_DIRECTION_TOP_BOTTOM | 3 | Print from the top to bottom |

### 2-1-22 Set / release Bold

| Code | Value | Description |
| --- | --- | --- |
| RT_BOLD_DIS | 0x00 | Release Bold print mode |
| RT_BOLD_EN | 0x01 | Set bold print mode |

### 2-1-23 Select paper cutting mode

| Code | Value | Description |
| --- | --- | --- |
| RT_CUT_PARTIAL | 0x31 | partial cutting(have a less bit retained) |
| RT_CUT_WITHFEED | 0x42 | Feed paper(cutting position + [n ×0.125mm]), And partial cutting(have a less bit retained). |

### 2-1-24 Set/Release invert printing mode

| Code | Value | Description |
| --- | --- | --- |
| RT_INVERT_DIS | 0 | Release invert printing |
| RT_INVERT_EN | 1 | set invert printing |

### 2-1-25 Port selection

| Code | Value | Description |
| --- | --- | --- |
| RT_CONNECTION_WIFI | 0x0000 | Wi-Fi |
| RT_CONNECTION_ETHERNET | 0x0001 | Ethernet |
| RT_CONNECTION_BLUETOOTH | 0x0002 | Bluetooth |

# 3.RTPrinter command control class definition (reference)

| Inherits from | NSObject |
| --- | --- |
| Confirms to |  |
| Framework | libWifiPrinter_SDK.a CFNetwork.framework |
| [Availability] | iOS 4.3 or higher |
| Declared | WifiPrinter.h |

## 3-1 Overview

This section mainly describes our Company's printer control method class definition.

## 3-2 Porperties

### 3-2-1 delegate

@property (readwrite) id<initWithDelegate> de

【Description】

This part is for the control function initialization

【Applicable version】

SDK 1.0.0 or higher

【Example】

@interface delegateTestClass : NSObject { //Declaration } @end

@implementation TestClass //process -(id) initWithDelegate : (id) de { //Implementation self = [super init]; if(self){ delegate = de; if(mCmdBuffer == NULL) mCmdBuffer = malloc(sizeof(Byte)*BUFFER_MAX_LEN); mIndex = 0; } return self; }

### 3-2-2 - (BOOL) InitPrinter

【Description】

This part is for the printer initialization

【Applicable version】

SDK 1.0.0 or higher

【Example】

Byte combyte[] = {0x1B, 0x40}; if(![asyncSocket isConnected]) retrurn false; NSData *data = [[NSData alloc] initWithBytes : combyte length : 2]; [self SendData : data]; return true;

### 3-2-3 - (void) HT

【Description】

Moves the print position to the next level anchor position (horizontal orientation)

HT

【Applicable version】

SDK 1.0.0 or higher

【Example】

mCmdBuffer[mIndex++] = 0x09;

### 3-2-4 - (void) LF

【Description】

Print the data form the butter area,and forward the the paper one line in accordance with the current line spacing

LF

【Note】

This command sets the print position to the beginning of the line

【Applicable version】

SDK 1.0.0 or higher

【Example】

mCmdBuffer[mIndex++] = 0x0A;

### 3-2-5 - (void) CR

【Description】

Print and carriage return	CR

CR

【Note】

- This command function is the same as LF’s function when allowing automatic document feed.

- The command is ignored when not allow automatic document feed.

- For serial interface mode, the command’s line paper feed function is ignored.

- Set the printing start position to the starting point of the line.

【Applicable version】

SDK 1.0.0 or higher

【Example】

mCmdBuffer[mIndex++] = 0x0D;

### 3-2-6 - (void) Selftest Print

【Description】

Print test test page

【Applicable version】

SDK 1.0.0 or higher

【Example】

mCmdBuffer[mIndex++] = 0x12; mCmdBuffer[mIndex++] = 0x54;

### 3-2-7 - (void) SetRightSpace:(Byte)Distance

@property (readwrite) Byte Distance

【Description】

Set Character right interval as [Distance *0.125 mm]

【Note】

- For Times the width mode, character right interval is twice wider than normal mode. If font size is enlarged, character right space is n times wider than normal model.

- The demand does not influence the set of Chinese character.

- Set value standard mode independently in every mode.

【Applicable version】

SDK 1.0.0 or higher

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x20; mCmdBuffer[mIndex++] = Distance;

### 3-2-8  - (void) SetAbsoluteprintPosition:(Byte)nL highByte:(Byte)nH

@property(readwrite) Byte  nL

@property(readwrite) Byte  nH

【Description】

This command sets the print position to the beginning of the line.

The space from the print position to the beginning of the line is [(nL + nH *256)*0.125 mm].

【Note】

- The set outside the specified print range is ignored.

- Use the horizontal motion units in standard mode.

【Applicable version】

SDK 1.0.0 or higher

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x24; mCmdBuffer[mIndex++] = nL; mCmdBuffer[mIndex++] = nH;

### 3-2-9  - (void) SetRelativeprintPosition:(Byte)nL highByte:(Byte)nH

@property(readwrite) Byte  nL

@property(readwrite) Byte  nH

【Description】

Take the current position as basis point, use horizontal or vertical motion units, set the beginning& ending print position.

The command set the print position from the current position to [(nL + nH *256)*0.125 mm].

【Note】

- Any set is ignored if outside the specified print range.

- If the space N is appointed to right : nL+ nH*256 = N

If N is appointed to left: (reverse), use the complement of 65536. nL+ nH*256 = 65536 – N.

【Applicable version】

SDK 1.0.0 or higher

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x5C; mCmdBuffer[mIndex++] = nL; mCmdBuffer[mIndex++] = nH;

### 3-2-10  - (void) Set Default Line Spacing

【Description】

Linespacing is selected to be 3.75 mm(30*0.125mm).

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x32;

### 3-2-11 - (void) Set Line Spacing:(Byte)Line Spacing

@property (readwrite) Byte Line Spacing

【Description】

This part is to set line spacing, as param * 0.125mm.

ESC  3  n

【Note】

- Line spacing can be set independently in standard mode and page mode.

- Use vertical motion units in standard mode(y).

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmd Buffer[mIndex++] = 0x1B mCmd Buffer[mIndex++] = 0x33; mCmd Buffer[mIndex++]= Set Line Spacing;

### 3-2-12  - (void) Set Left Start Spacing:(Byte)nL highByte:(Byte)nH

@property(readwrite) Byte  nL

@property(readwrite) Byte  nH

【Description】

Left blank is set as[(nL + nH *256)*0.125 mm].

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmd Buffer[mIndex++] = 0x1D; mCmd Buffer[mIndex++] = 0x4C; mCmd Buffer[mIndex++] = nL; mCmd Buffer[mIndex++] = nH;

### 3-2-13  - (void) Set Area Width:(Byte)nL highByte:(Byte)nH

@property(readwrite) Byte  nL

@property(readwrite) Byte  nH

【Description】

The width of print range is set as [(nL + nH *256)*0.125 mm].

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmd Buffer[mIndex++] = 0x1D; mCmd Buffer[mIndex++] = 0x57; mCmd Buffer[mIndex++] = nL; mCmd Buffer[mIndex++] = nH;

### 3-2-14  - (void) Set Character Printmode:(Byte)Character Print mode

@property(readwrite) Byte  Character Print mode

【Description】

Select print mode by the specified parameter “Character Print mode”. The parameter n is defined as follows:

#define RT_FT_DEFAULT                0x00

#define RT_FT_FONTA                   0x00

#define RT_FT_FONTB                   0x01

#define RT_FT_BOLD                    0x08

#define RT_FT_DHEIGHT                0x10

#define RT_FT_DWIDTH                 0x20

#define RT_FT_UNDERLINE              0x80

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmd Buffer[mIndex++] = 0x1D; mCmd Buffer[mIndex++] = 0x21; mCmd Buffer[mIndex++] = RT_FT_DHEIGHT;

### 3-2-15  - (void) Set Underline:(Byte)UnderLine En

@property(readwrite)Byte  UnderLineEn

【Description】

Parameter “UnderLineEn” determines underline parameter, which is defined as follows：

#define RT_UNDERLINE_DIS             0

#define RT_UNDERLINE_1               1

#define RT_UNDERLINE_2               2

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmd Buffer[mIndex++] = 0x1B; mCmd Buffer[mIndex++] = 0x2D; // Set underline mode thickness is 2 point mCmd Buffer[mIndex++] = RT_UNDERLINE_1;

### 3-2-16  - (void) Add Bold:(Byte)BoldEn

@property(readwrite)Byte  BoldEn

【Description】

This section is used to set / release bold print.

Parameter BoldEn  is defined as follows：

#define RT_BOLD_DIS                  0x00

#define RT_BOLD_EN                   0x01

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x45; // Set the bold print mCmdBuffer[mIndex++] = RT_BOLD_EN;

### 3-2-17  - (void) SetCharacterFont:(Byte)Font

@property(readwrite)Byte  Font

【Description】

This section is used to select the character for font A or B.

Parameter Font  is defined as follows：

#define RT_FONT_A                 0x00

#define RT_FONT_B                 0x01

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x4D; // Font B（9x17） mCmdBuffer[mIndex++] = RT_FONT_B;

### 3-2-18  - (void) SetRotate:(Byte)RotateEN

@property(readwrite)Byte  RotateEN

【Description】

Set/release 90 ° clockwise rotation.

Parameter RotateEN  is defined as follows:

#define RT_CLOCKWISE_DIS             0

#define RT_CLOCKWISE_EN              1

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmd Buffer[mIndex++] = 0x1B; mCmd Buffer[mIndex++] = 0x56; // Set the 90 ° clockwise rotation mode mCmd Buffer[mIndex++] = RT_CLOCKWISE_EN;

### 3-2-19  SetAlignMode

@property(readwrite)Byte  AlignMode

【Description】

This part is for selecting alignment.

Parameter AlignMode is defined as follows:

#define RT_ALIGNMENT_LEFT            0

#define RT_ALIGNMENT_CENTER          1

#define RT_ALIGNMENT_RIGHT           2

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x61; //Align Left mCmdBuffer[mIndex++] = RT_ALIGNMENT_LEFT;

### 3-2-20  - (void) SetInvert Print:(Byte)Invert ModeEn

@property(readwrite)Byte  InvertModeEn

【Description】

This section is mainly used to set / release reverse print mode. 

When the lowest valid position of n is 0, reverse print mode turn off.

When the lowest valid position of n is 1, reverse print mode turn on.

Parameter InvertModeEn is defined as follows:

#define RT_INVERT_DIS				0

#define RT_INVERT_EN				1

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x7b; mCmdBuffer[mIndex++] = RT_INVERT_EN;

### 3-2-21  - (void) Set Zoom:(Byte)Font Enlarge

@property(readwrite) Byte  FontEnlarge

【Description】

This section is used to set the character size.

GS  !  n

【Note】

- The command set with 0-3 character height, 4-7 to set the character width.

- The command is valid for all the characters(alphanumeric characters and Chinese characters)except HRI characters.

- When the characters in different sizes amplify in a row, all the characters in a row along the baseline align.

- In standard mode, the vertical direction is the paper feed direction. However, when the characters rotate 90 ° clockwise, the relationship between the vertical and the horizontal direction is reversed.

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmd Buffer[mIndex++] = 0x1D; mCmd Buffer[mIndex++] = 0x21; mCmd Buffer[mIndex++] = FontEnlarge;

### 3-2-22  - (void) Set BlackReverse Print:(Byte)BlackReverse En

@property(readwrite) Byte  BlackReverseEn

【Description】

The section is used to set/release black reverse print mode.

Parameter InvertModeEn is defined as follows：

#define RT_INVERSE_DIS               0x00

#define RT_INVERSE_EN                0x01

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmdBuffer[mIndex++] = 0x1D; mCmdBuffer[mIndex++] = 0x42; // Set Black Reverse Print Mode mCmdBuffer[mIndex++] = RT_INVERSE_EN; // Release Black Reverse Print Mode mCmdBuffer[mIndex++] = RT_INVERSE_DIS;

### 3-2-23  - (void) OpenDrawer:(Byte)DrawerNumber startTime:(Byte)pulseStartTime endTime:(Byte)PulseEndTime

@property(readwrite) Byte  DrawerNumber

@property(readwrite) Byte  pulseStartTime

@property(readwrite) Byte  PulseEndTime

【Description】

DrawerNumber  = 0, 1, 48, 49

0 <=  pulseStartTime <= 255

0 <=  PulseEndTime <= 255

【Description】

Send an impulse to specified connected pins.

On time= pulseStartTime x 2 millisecond

Off time= PulseEndTime x 2 millisecond

m =0/48 Impulse sent to cash drawer and then output pin 2;

m=1/49 Impulse sent to cash drawer and then output pin 5.

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x70; mCmdBuffer[mIndex++] = DrawerNumber; mCmdBuffer[mIndex++] = pulseStartTime; mCmdBuffer[mIndex++] = PulseEndTime;

### 3-2-24  - (void) Cut Paper

【Description】

Full cut paper

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x69;

### 3-2-25  - (void) partial Cut Paper

【Description】

Partial cut paper

【Applicable version】

SDK 1.0.0 or higher.

【Example】

mCmdBuffer[mIndex++] = 0x1B; mCmdBuffer[mIndex++] = 0x6D;

### 3-2-26     ① - (void) Feed And Cut Paper:(Byte)Cut Mode

### ②- (void) Feed And Cut Paper:(Byte)Cut Mode feed:(Byte)Feed Distance

【Range】

- Cut Mode：RT_CUT_PARTIAL

- Cut Mode：RT_CUT_WITHFEED

0 <= FeedDistance <= 255

【Description】

Select cut paper mode and cutting.

【Syntax】

- (void) FeedAndCutPaper:(Byte)CutMode

- (void) FeedAndCutPaper:(Byte)CutMode feed:(Byte)FeedDistance

【Example】

- (void) FeedAndCutPaper:(Byte)CutMode Parameter: CutMode：is defined as follows： #define RT_CUT_PARTIAL 0x31 Len ：Data length

- (void) Feed And Cut Paper:(Byte)Cut Mode feed:(Byte)Feed Distance Parameter： Cut Mode：is defined as follows： #define RT_CUT_WITHFEED 0x42 Feed Distance：Feed points

### 3-2-27  Add Bar Code Print

@property(readwrite)int  codeType

@property(readwrite)(NSData *)  data

【Description】

This part is used to select barcode system and print barcode.

GS  k  m  n  d1  …  dn

【Notice】

- Parameter Code Type is used to select the barcode system, which is the parameter m in command.

- Data for input parameters d1…dn, parameter n is calculated based on the input parameter’s number of d1…dn

- n specifies the number of bytes of bar code data, and the printer will start from the next n-byte character data as bar code data processing.

- If n is outside the specified range, the printer stops processing the command, and subsequent data as normal data.

- If d is outside the specified range, the printer just feed and deal with the subsequent data as normal data.

- The command feeds according to the requirements of the bar code printing, regardless of the line spacing set by ESC 2 or ESC 3.

- Only when the print buffer is no data, the command is valid. Otherwise the printer deals with the subsequent data as common ones.

- After printing the bar code, this command will set print position at the beginning of the line.

- The command is not effected by print mode(bold overlap, underline, character size, reverse print or characters 90 ° rotation, etc.),except upside-down printing mode.

- The command is not effected by print mode(bold overlap, underline, character size, reverse print or characters 90 ° rotation, etc.),except upside-down printing mode.

【Applicable Version】

SDK 1.0.0 or higher

【Example】

//******Print 1D Barcode**********// - (void) Add BarCode Print: (int)code Type :(NS Data *)data { Switch (code Type){ case RT_BCS_UPCA: [self UPCA: data]; break; case RT_BCS_UPCE: [self UPCE: data]; break; case RT_BCS_EAN13: [self EAN13:data]; break; case RT_BCS_EAN8: [self EAN8:data]; break; case RT_BCS_CODE39: [self CODE39:data]; break; case RT_BCS_ITF: [self ITF: data]; break; case RT_BCS_CODABAR: [self CODEBAR: data]; break; case RT_BCS_CODE93: [self CODE93:data]; break; case RT_BCS_CODE128: [self Code128_B:data]; break; default : break; } }

### 3-2-28 Begin Print Image

@property (read write) (UIImage *) image

【Description】

This part is used to handle bitmap data.

【Applicable Version】

SDK 1.0.0 or higher

【Example】

CG Image Ref image ref = [image CG Image]; CG Color Space Ref color space = CG Color Space Create Device Gray(); CG Context Ref cg cnt = CG Bit map Context Create (image Data, width, height, …); CG Rect there ct = CG Rect Make (0, 0, width, height); CG Con text Draw Image (cg cnt, there ct, image ref); CG Color Space Release(color space); CG Context Release (cg cnt); // Parse picture, get the bitmap data … //Bitmap data processing … // Send print pictures preamble command Byte start[] = {0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x1B, 0x40, 0x1B, 0x33, 0x00}; NS Data *data Start = [NS Data data With Bytes : start length : 13]; [self Begin]; [self Import Data : data Start Len : [data Start length]; [self Excute]; [self Clear Data]; //Send picture data [self Import Data : final Data Len : [final Data length]]; [self Excute]; [self Clear Data]; //Transmit complete demand Byte end[] = {0x1d, 0x4c, 0x1f, 0x00}; NS Data *data End = [NS Datra data With Bytes : end length : 4]; [self Import Data : data End Len : [data End length]]; [self Excute]; [self Clear Data];

## 3-3  Instance Methods

### 3-3-1  Begin

【Description】

This is used to initialize the printer and configure application environment.

【Syntax】

-（void） Begin

【Applicable Version】

SDK 1.0.0 or higher

【Example】

[self Wake Up Printer]; [self Init Printer]; //Clear the printer buffer [self Clear Data];

### 3-3-2 Excute

【Description】

This part is for execution of processing data.

【Applicable Version】

SDK 1.0.0 or higher

【Example】

NS Data *data = [[NS Data alloc] init WithBytes : mCmd Buffer length : mIndex]; [self send Data : data] M Index = 0;

### 3-3-3 Import Data

【Description】

This part is for inputting printing data.

【Syntax】

-（void）Import Data : (NS Data *)data  Len : (int) len

-（void）Import Data : (NS String *)data

【Example】

-（void）Import Data : (NS Data *)data Len : (int) len Parameter: data ：content of data Len ：length of data

-（void）Import Data : (NS String *)data Parameter： data ：print character string data

### 3-3-4  Clear Data

【Description】

Execute Clear Data() for data buffer area after execution of Execute printing.

【Syntax】

-（void）Clear Data

### 3-3-5  btn Connect

【Description】

This part is used for connect/disconnect printer connection.

【Syntax】

-（IB Action）btn Connect

-（void）on Connected

-（void）on Disconnected

【Example】

//on Connected if(is Succeed) { [btn set Title : LANG(@”disconnect”) for State : UI Control State Normal]; Lab Status. text = LANG(@”connect”); } else { MSGBOX(LANG(@”connection failed”)); }

//on DisConnected if(is Succeed) { [btn set Title : LANG(@”connection”) for State : UI Control State Normal]; Lab Status. text = LANG(@”disconnected”); } else { MSGBOX(LANG(@”disconnection failed”)); }

### 3-3-6  btnPrint

【Description】

This part is for testing simple printer functions.

【Syntax】

-（IB Action）btn Print

【Applicable Version】

SDK 1.0.0 or higher

【Example】

if(sw Double Width. Is On){ [printer Set Zoom : (Byte)0x10]; } if(sw Double Height. isOn){ [printer Set Zoom : (Byte)0x01]; } If (sw Under Line. is On){ [printer Set Underline : (Byte)0x02]; } If (sw Bold. is On){ [printer Add Bold : (Byte)0x01]; } If (sw Small Font. Is On){ [printer Set Character Font : (Byte)0x01]; } If (sw To White. is On){ [printer Add Inverse : (Byte)0x01]; }

### 3-3-7  btn Preferences Click

【Description】

This part is to select the test program print characters, pictures, bar codes, invoices or forms.

【Syntax】

-（IB Action）btn Preferences Click

-（IB Action）btn Print T ext

-（IB Action）btn Print Pic

-（IB Action）btn Print Barcode

-（IB Action）btn Print Invoice

-（IB Action）btn Print Table

【Applicable】

SDK 1.0.0 or higher

【Example】

//btn Preferences Click Preferences *pref = [[Preferences alloc]init]; pref. printer = self. printer; App Delegate *app = [[UI Application shared Application]delegate]; [app. Nav Controller push View Controller : pref animated : YES];

//btn Print Text if(IS_LANG_EN) { [printer Import Data : @ “abcd!!!!\n”]; } else { [printer Import Data : @”test！！！\n”]; }

//btn PrintPic UI Image *img = [UI Image image Named : @”**.bmp”]; [printer Begin Print Image : img];

//btn Print Barcode …

//btn Print Invoice …

//btn Print Table …

# 4.Agreement Involved

| Inherits from | NS Object |
| --- | --- |
| Confirms to |  |
| Framework | Lib Wifi Printer_ SDK.a CF Network. framework |
| 【Availability】 | iOS 4.3 or higher |
| Declared | Wifi Printer Delegate. h |

## 4-1 Outline

This part is for WiFi connection protocol.

## 4-2 Instance Methods

### 4-2-1 connect Host

【Description】

This part is for Wifi connection.

【Syntax】

-（void）connect Host : （NS String *）host PORT : （UInt16）prot

-（void）on Socket : (Async Socket *)sock did Connect To Host : (NS String *)host port : (UInt16)port

【Applicable Version】

SDK 1.0.0 or higher

【Example】

//connect Host if(![async Socket connect To Host : host on Port : port with Timeout : TIMEOUT error : &err]) { NSLog(@”Error: %@”, err); } [NS Thread detach New Thread Selector : @selector(thread Func:) to Target : self with Object : @”CONN”];

//did Connect To Host NS Log(@”connected.”); if([delegate responds To Selector : @selector(on Connected:)]) { [delegate on Connected : TRUE]; }

### 4-2-2  disconnect

【Description】

This part is for WiFi disconnection.

【Syntax】

-（void）disconnect

【Applicable Version】

SDK 1.0.0 or higher

【Example】

B DisconnOK = FALSE; If ([async Socket is Connected]) { … } Else { B DisconOK = TRUE; if([delegate responds To Selector : @selector(on Disconnected:)]) { [delegate on Disconnected : TRUE]; } }

# 5.Appendix

【Part of product catalog】

| Model | Port | Type |
| --- | --- | --- |
| RP80 | Parallel/Serial/USB/Ethernet | Thermal Receipt Printer |
| RP58 | USB/Serial/Parallel | Thermal Receipt Printer |
| RP58E | USB/Serial/Parallel | Thermal Receipt Printer |
| RP80W | WIFI | WIFI printer |
| RPP-02 | Original IR，VIR，IrDA（Ir COMM）RS232 interface，blue tooth | Portable printer |
| RP203 | Serial（RS-232C,TTL） | Panel thermal printer |
| RP07 | Serial（RS-232C,TTL） | Panel thermal printer |
| RP76 | Serial/Parallel/USB/cash port | Impact printer |
| RP76II | Serial/Parallel/USB/cash port | Impact printer |
| PM628 |  | Printer module |
| PM65C |  | Printer module |
| PM532 |  | Printer module |
| CT628 |  | Paper cutter |
| CB628 |  | Control board |
| CB1245 |  | Control board |